IOT Material

COMMANDS

- Creating new file in VMWARE

Nano filename.py

- Running file

python3 filename.py

- Creating sqlite db

sqlite3 databasename

Thingspeak

Sign in -> verification done in gmail -> continue (click in thingspeak) ->after successfully signed in ->channels ->my channel->new channel->give name and feild name

Save channel->copy chanel id value

Device tab->mqtt->select channel

-> Add device (last option) -> download credinetial in plain text

Support tab-> examples->search->mqtt basic->scroll down and select last to fourth(publish using web socket in raspberry pi)

For installing paho:

pip install paho-mqtt

import paho.mqtt.publish as publish

import psutil

import string

channel_ID = "1799852"

mqtt_host = "mqtt3.thingspeak.com"

mqtt_client_ID = "BDIiNR4CIxcPDBQzBBYLNwg"

mqtt_username = "BDIiNR4CIxcPDBQzBBYLNwg"

mqtt_password = "qbNDlRyj3xyUqbkEkg49okTV"

t_transport = "websockets"

t_port = 80

topic = "channels/" + channel_ID + "/publish"

while (True):

# get the system performance data over 20 seconds

cpu_percent = psutil.cpu_percent(interval=20)

ram_percent = psutil.virtual_memory().percent

#build the payload string

payload = "field1=" + str(cpu_percent) + "&field2=" + str(ram_percent)

#attempt to publish this data to the topic

try:

print("Writing Paylod = ", payload," to host: ", mqtt_host, " clientID= ",mqtt_client_ID, "username= ",mqtt_username, " PWD ",mqtt_password)

publish.single(topic, payload, hostname=mqtt_host, transport=t_transport, port=t_port, client_id=mqtt_client_ID, auth={'username':mqtt_username,'password':mqtt_password})

except (keyboardInterrupt):

break

except Exception as e:

print (e)

Firebase.com

Sign in -> click on get started-> create/add project->write project name ->disable google analytics in the next step->continue->build.->realtime database

Create database->select singapore->

Click next->select start in test mode

Click Enable

(database is created on cloud)

For keys-: go to project overview->select web </> icon-> add name(same as project name)->register app->script will be displayed with id and keys.

.py program(shruti account)

- Run this command

sudo pip3 install pyrebase

import pyrebase

Config = {

"apiKey": "AIzaSyDJ7Ir1QewmxyTZjXjO8meu6LtihcsrpuQ",

"authDomain": "mydemo-af3e4.firebaseapp.com",

"databaseURL": "https://mydemo-af3e4-default-rtdb.asia-southeast1.firebasedatabase.app",

"projectId": "mydemo-af3e4",

"storageBucket": "mydemo-af3e4.appspot.com",

"messagingSenderId": "628906293168",

"appId": "1:628906293168:web:578a9db8b337b0f157c875",

"measurementId": "G-TL6WMB3BQG"

};

firebase = pyrebase.initialize_app(Config);

storage =firebase.storage()

database =firebase.database()

a = 6

b = 60

print (a)

database.child("DB _object_name")

data = {"key1" :a,"key2" :b}

database.set(data)

no1 =33

no2 =33

database.child("DB _object_name2")

data1 = {"num1" :no1,"num2" :no2}

print (no1)

database.set(data1)

IOT_MQTT_to_rasbearyPI_publish_subscribe

import paho.mqtt.client as mqtt

def on_message(client, userdata, msg):

print(msg.topic+" "+str(msg.payload))

client.publish(topic="it", payload="TestingPayload", qos=1, retain=False)

#broker_url = "mqtt.eclipse.org"

broker_url = "broker.emqx.io"

broker_port = 1883

client = mqtt.Client()

client.on_message = on_message

client.connect(broker_url, broker_port)

client.subscribe("TestingTopic", qos=0)

#client.publish(topic="TestingTopic1", payload="TestingPayloadh1" ,qos-0, retain$

#print(msg.topic+" "+str(msg.payload))

client.loop_forever()

Program 2

import paho.mqtt.client as mqtt

def on_message(client,userdata,msg):

print(msg.topic+" "+str(msg.payload))

client.publish(topic="it",payload="Testing payload 7",qos=1,retain=False)

broker_url = "broker.emqx.io"

broker_port = 1883

client=mqtt.Client()

client.on_message=on_message

client.connect(broker_url,broker_port)

client.subscribe("TestingTopic",qos=0)

#client.publish(topic="TestingTopic1",payload="testing payload",qos=1,retain=False)

client.loop_forever()

IOT_sqlite_to_mqtt_publish_and_sunscribe

external_exam.py

import sqlite3

import time

import datetime

import paho.mqtt.client as mqtt

broker_url="broker.emqx.io"

broker_port=1883

client=mqtt.Client()

client.connect(broker_url,broker_port)

connection = sqlite3.connect("hospitalitydb.db")

crsr = connection.cursor()

sql_command = """ CREATE TABLE IF NOT EXISTS tbl_demo(id INTEGER PRIMARY KEY AUTOINCREMENT,name VARCHAR(50),age INTEGER,date DATE) """

crsr.execute(sql_command)

print("table created...")

for i in range(1):

print("Enter Name : ")

name=input()

print("Enter Age : ")

age=input()

date = datetime.datetime.now()

print(name)

crsr.execute("INSERT INTO tbl_demo(name,age,date) values(?,?,?)",(name,age,date))

connection.commit()

crsr.execute("select * from tbl_demo")

allrecord=crsr.fetchall()

print(allrecord)

crsr.execute("select name from tbl_demo")

ans=crsr.fetchall()

i=ans

for i in range (len(ans)):

print(ans[i])

client.publish(topic="it",

payload="name: " + str(ans[i]),

qos=0,

retain=False)

print("msg published...")

Displaydata.py

import sqlite3

connection = sqlite3.connect("hospitalitydb.db")

crsr = connection.cursor()

crsr.execute("select * from hospital")

ans=crsr.fetchall()

print(ans)

I2C code

Arduino code

#include<Wire.h>

int i2cData =0x56;

//LED on pin 13

const int ledPin=13;

int c;

void setup(){

//join I2C bus as slave with address 8

Wire.begin(0x8);

Serial.begin(9600);

Wire.onReceive(receiveEvent);

Wire.onRequest(sendData);

pinMode(ledPin,OUTPUT);

digitalWrite(ledPin,LOW);

}

void receiveEvent(int howMany)

{

while(Wire.available())

{

c=Wire.read();

digitalWrite(ledPin,c);

}

}

void loop()

{

delay(1000);

if(c<0x02){

Serial.println(c);

c=0x02;

}

}

void sendData()

{

Wire.write(i2cData);

}

Raspberry file

from smbus import SMBus

addr= 0x8

bus = SMBus(1)

numb = 1

print("Enter 1 for ON or 0 for OFF")

while numb==1:

ledstate = input(">>>> ")

if ledstate == "1":

bus.write_byte(addr,0x1)

elif ledstate == "0":

bus.write_byte(addr,0x0)

else:

numb=0

SPI

Arduino code

#include <SPI.h>

char buf[100];

byte c=0,b=0;

volatile byte pos;

volatile boolean processing;

byte i;

void setup(void)

{

Serial.begin(115200); //uart speed

pinMode(MISO,OUTPUT);

pinMode(MOSI,INPUT);

SPCR |= _BV(SPE);

pos=0;

processing=false;

SPI.attachInterrupt();

}

ISR(SPI_STC_vect){

c= SPDR;

processing = true;

}

void loop(void){

if(processing){

Serial.println(c);

processing = false;

SPDR =i;

i= i+1;

}

}

Raspberry py code

import spidev

import time

spi = spidev.SpiDev(0,0)

spi.open(0,0)

msg = 0xAA

spi.max_speed_hz = 115200

while 1:

spi.writebytes([2,4])

y = spi.readbytes(1)

print (y)

time.sleep(0.5)

Ultrasonic

wire confi:

vcc => 5v

trig =>gpio21 =>40 no pin

echo =>gipio20 =>38no pin

gnd => 39 no pin

import RPi.GPIO as GPIO

import time

TRIG=21

ECHO=20

GPIO.setmode(GPIO.BCM)

while True:

print("distance measurement in progress")

GPIO.setup(TRIG,GPIO.OUT)

GPIO.setup(ECHO,GPIO.IN)

GPIO.output(TRIG,False)

print("waiting for sensor to settle")

time.sleep(0.2)

GPIO.output(TRIG,True)

time.sleep(0.00001)

GPIO.output(TRIG,False)

//high pulse=1

//low pulse=0

while GPIO.input(ECHO)==0:

pulse_start=time.time()

while GPIO.input(ECHO)==1:

pulse_end=time.time()

pulse_duration=pulse_end-pulse_start

distance=pulse_duration*17150

distance=round(distance,2)

print("distance:",distance,"cm")

time.sleep(2)

PIR

pin Config:

- pin =Grd

+ pin = 5 v

data (middle) = gpio18 =12 no

led = 1 ->grd

2=> gpio24 pin no18

import RPi.GPIO as GPIO

import time

GPIO.setmode(GPIO.BCM)

GPIO.setup(24,GPIO.OUT)

GPIO.setup(18,GPIO.IN)

while(True):

myin=GPIO.input(18)

if myin==True:

print("Motion Detected")

time.sleep(1)

GPIO.output(24,True)

time.sleep(0.5)

GPIO.output(24,False)

time.sleep(0.5)

IR

GND-pin6

Vcc-pin4

Data-pin8 (GPIO14)

import RPi.GPIO as IO

#import time

IO.setwarnings(False)

IO.setmode(IO.BOARD)

IO.setup(3,IO.OUT)

IO.setup(8,IO.IN)

while(True):

if(IO.input(8) == True):

print ("Obstacle Detected")

IO.output(3,True)

else:

print ("Obstacle not Detected")

IO.output(3,False)

LDR

Arduino code

int ldr;

void setup() {

// put your setup code here, to run once:

pinMode(13,OUTPUT);

Serial.begin(9600);

}

void loop() {

// put your main code here, to run repeatedly:

ldr = analogRead(A0);

if(ldr < 50){

digitalWrite(13,HIGH);

}

else{

digitalWrite(13,LOW);

}

Serial.println(ldr);

delay(100);

}

USB

Error commands

sudo chmod 666 /dev/ttys0

sudo chmod 666 dev/ttyACM0

rasberry-arduino

(8pin)UART tx-rx

(10pin)rx-tx

gnd-gnd

//single pc

import serial

#if __name__=='__main__':

ser=serial.Serial('/dev/ttyACM0',9600,timeout=1)

ser.flush()

while True:

if ser.in_waiting>0:

line=ser.readline().decode('utf-8').rstrip()

#rstrip for storing whole line in buffer 4-500 characters

print(line)

//2px tx_rx

import serial

#if __name__=='__main__':

ser=serial.Serial('/dev/ttyACM0',9600,timeout=1)

ser.flush()

while True:

if ser.in_waiting>0:

line=ser.readline().decode('utf-8').rstrip()

#rstrip for storing whole line in buffer 4-500 characters

print(line)

//Arduino code

void setup()

{

Serial.begin(9600);

}

3T75c8qU

void loop()

{

Serial.println("Hello From Arduino!");

delay(1000);

}

UART with tx-rx

| Arduino | RPi |
| --- | --- |
| Tx | 8 |
| Rx | 10 |
| Gnd | 6 |

import serial

from time import sleep

ser=serial.Serial('/dev/ttyS0',9600)

while True:

received_data=ser.read()

sleep(0.03)

data_left=ser.inWaiting()

received_data+=ser.read(data_left)

print(received_data)

Note : run sudo chmod 666 /dev/ttyS0 to give persmission to use ttyS0 before executing code

USB UART with RPi direct communication

| Arduino | RPi |
| --- | --- |
| Tx | Tx |
| Rx | Rx |
| Gnd | Gnd |

import serial

if name=='main':

ser=serial.Serial('/dev/ttyACM0',9600,timeout=1)

ser.flush()

while True:

if ser.in_waiting>0:

line=ser.readline().decode('utf-8').rstrip()

print(line)

Note : run sudo chmod 666 /dev/ttyACM0 to give persmission to use ttyS0 before executing code

Arduino Code For USB UART

void setup()

{

Serial.begin(9600);

}

void loop()

{

Serial.println("Hello From Arduino!");

delay(1000);

}

------------------------------------------------------------------------------------------------------------------

- Connect IR sensor with RPi .sense object using IR sensor and update timing in SQLite db also publish msg “object detected” on MQTT topic name :: “IR_Sensor” and display time on screen

| IR | RPi |  |
| --- | --- | --- |
| Data | 3 | (without using LED) |
|  | 8 | (while using LED) |
| Vcc | 2 |  |
| Gnd | 6 |  |

If using LED

| LED | RPi |
| --- | --- |
| - | ground |
| + | 3 |
| import RPi.GPIO as IO |  |
| import time |  |
| import sqlite3 |  |
| import paho.mqtt.client as mqtt |  |

connection = sqlite3.connect('test.db')

cursor = connection.cursor()

cursor.execute('''CREATE TABLE irTB ( sensorData VARCHAR(100), currentTime TIMESTAMP);''')

insertQuery = """INSERT INTO irTB  VALUES (?, ?, ?);"""

IO.setwarning(False)

IO.setmode(IO.BOARD)

IO.setup(8,IO.IN)

IO.setup(3,IO.OUT)

while 1:

if(IO.input(8)==True):

print("Obstacle Detected!!")

cursor.execute(insertQuery, (“Obstacle Detected”, currentDateTime))

connection.commit()

broker_url = "broker.emqx.io"

broker_port = 1883

client = mqtt.Client()

client.connect(broker_url,broker_port)

client.publish(topic="IR_Sensor", payload=”object detected”, qos=0, retain= False)

IO.output(3,True)

else

print("Obstacle Not Detected")

IO.output(3,False)

cur.execute("select * from irTB ");

ans=cur.fetchall();

for row in ans:

print(ans);

connection.close()

2.     Sense light intensity using LDR on arduino board and send sensed light intensity value to RPi using i2c bus on RPi received content through i2c bus is update in SQLite DB and display on screen

Requirement : Arduino, LDR, RPi, cable I2c :

| Arduino | RPi |
| --- | --- |
| A4-SDA | 3-SDA |
| A5-SCL | 5-SCL |
| GND | 20 |

LDR : connects with arduino because RPi has no analog convertor pin

| LDR | Arduino |
| --- | --- |
| 2 joint leg | A0 |
| Register | GND |
| LDR | 5v |

LDR arduino code : (Slave)

#include<Wire.h>

int ldr; //set A0 (Analog Input) for LDR

void setup(){

//join i2c bus as slave with address 8

Wire.begin(0x8);

Serial.begin(9600);

//call receiveEvent when data received

Wire.onReceive(receiveEvent);

//call get request from master

Wire.onRequest(sendData);

pinMode(13,OUTPUT);

digitalWrite(13,LOW);

//fn that executes whenever data is received from master void receiveEvent (int howMany){

while(Wire.available()){

c=Wire.read();  //receive byte as a character

digitalWrite(13,c);

}

}

void loop(){

ldr=analogRead(A0);	//reads the value of LDR(light)

if(ldr<50)

{	digitalWrite(13,HIGH);	}

else

{	digitalWrite(13,LOW);	}

Serial.println(ldr);

delay(100);

//prints the value of ldr to Serial monitor

} //Serial.println(value); void sendData(){

Wire.write(ldr);

}

I2CPY : (Master)

from smbus import SMBus

import time

import sqlite3

conn=sqlite3.connect("testDB.db");

cur=conn.cursor();

conn.execute('''CREATE TABLE ldrTB(LDRDATA TEXT NOT NULL);''')

conn.commit();

addr = 0x8

bus = SMBus(1)

print("Enter 1 for ON or 0 for OFF")

while 1:

- = bus.read_byte_data(addr,0x1) print (y)

cur.execute("’INSERT INTO ldrTB(LDRDATA) VALUES(?)’",(y)); conn.commit()

cur.execute("’select * from ldrTB’");

ans=cur.fetchall();

for row in ans:

print(ans);

conn.commit()

conn.close()

2nd program

py

from smbus import SMBus

import time

import sqlite3

conn=sqlite3.connect("testDB.db");

cur=conn.cursor();

conn.execute('''CREATE TABLE finaltb(LdrSignal TEXT NOT NULL);''')

conn.commit();

addr = 0x8

bus = SMBus(1)

print("Enter 1 for ON or 0 for OFF")

while 1:

y = bus.read_byte_data(addr,0x1)

print (y)

cur.execute("’INSERT INTO finaltb(LdrSignal) VALUES(?)’",(y,));

conn.commit()

cur.execute("’select * from finaltb’");

ans=cur.fetchall();

for row in ans:

print(ans);

conn.commit()

conn.close()

Arduino

#include<Wire.h>

int Idr;

int c;

int i2cData = 0x56;

const int ledPin = 13;

void setup() {

Wire.begin(0x07);

Wire.onReceive(receiveEvent);

Wire.onRequest(sendData);

// put your setup code here, to run once:

//pinMode(ledPin,OUTPUT);

pinMode(13,OUTPUT);

digitalWrite(13,LOW);

Serial.begin(9600);

//pinMode(3,OUTPUT);

}

//Function that executes whenever data is received from master

void receiveEvent(int howMany)

{

while(Wire.available())

{

//Loop through all but the last

c = Wire.read();

digitalWrite(ledPin,c);

}

}

void loop() {

// put your main code here, to run repeatedly:

Idr=analogRead(A0);

//Wire.write("A");

if(Idr<50)

{

digitalWrite(13,HIGH);

}

else

{

digitalWrite(13,LOW);

}

Serial.println(Idr);

delay(100);

}

void sendData()

{

Wire.write(Idr);

}

3.   Connect ultrasonic sensor with RPi. Sense object distance and update distance and timing in

SQLite DB also publish measured distance value on MQTT topic name :: “ultra_Sensor” and display on screen

| Ultrasonic | RPi |
| --- | --- |
| ECHO | 38(GPIO 20) |
| TRIG | 40(GPIO 21) |
| VCC | 2 |
| GND | 39 |
| import RPi.GPIO as GPIO |  |
| import time |  |
| import sqlite3 |  |
| import paho.mqtt.client as mqtt |  |
| TRIG=16 |  |
| ECHO=20 |  |

connection = sqlite3.connect('test.db')

cursor = connection.cursor()

cursor.execute('''CREATE TABLE ultraTB (sensorData VARCHAR(100), currentTime TIMESTAMP);''')

insertQuery = """INSERT INTO ultraTB VALUES (?, ?, ?);"""

GPIO.setmode(GPIO.BCM)

while True:

print("distance measurement in progress");

GPIO.setup(TRIG,GPIO.OUT)

GPIO.setup(ECHO,GPIO.IN)

GPIO.output(TRIG,False)

print("waiting for sensor to settle");

time.sleep(0.2)

GPIO.output(TRIG,True)

time.sleep(0.00001)

GPIO.output(TRIG,False)

while GPIO.input(ECHO)==0:

pulse_start=time.time()

while GPIO.input(ECHO)==1:

pulse_end=time.time()

pulse_duration=pulse_end-pulse_start

dist=pulse_duration*17150

dist=round(dist,2)

print("distance:",dist,"cm")

time.sleep(0.1)

cursor.execute(insertQuery, (dist, currentDateTime))

connection.commit()

broker_url = "broker.emqx.io"

broker_port = 1883

client = mqtt.Client()

client.connect(broker_url,broker_port)

client.publish(topic="UltraMP", payload=dist, qos=0, retain= False)

cur.execute("select * from ultraTB ");

ans=cur.fetchall();

for row in ans:

print(ans);

connection.close()

4.  Sense light intensity using LDR on arduino board and send sensed light intensity value to RPi using SPI bus on RPi received content through SPI bus is update in SQLite DB and display on screen

Requirement : Arduino, LDR, RPi, cable SPI :

| Arduino | RPi |
| --- | --- |
| 11 | 19-MOSI |
| 12 | 21-MISO |
| 13 | 23-SCLK |
| GND | 25-ground |

LDR : connects with arduino because RPi has no analog convertor pin

| LDR | Arduino |
| --- | --- |
| 2 joint leg | A0 |
| Register | GND |
| LDR | 5v |

5 .    Use virtual box to create SQLite DB with name manualrollno DB. Insert following fields in DB table “Sequence no, name, age, gender” manually. Insert atleast 5 to 6 records in DB. Once

DB record entry completed then access it one by one and publish on topic name :: Virtual_A and display on screen

import paho.mqtt.client as mqtt

import sqlite3

connection=sqlite3.connect("manualrollno.db");

cur=connection.cursor();

create_tbl="""CREATE TABLE IF NOT EXISTS TEST(sequenceno INT, name TEXT, age INT, gender TEXT)"""

cur.execute(create_tbl);

connection.commit();

//insert 5-6 data manually

broker_url = "broker.emqx.io"

broker_port = 1883

client = mqtt.Client()

client.connect(broker_url, broker_port)

cur.execute("select * from TEST");

ans=cur.fetchall();

for row in ans:

print(ans);

client.publish(topic=" Virtual_A ",payload=ans,qos=0,retain=False)

connection.commit();

client.loop_forever()

6.  Connect PIR sensor with RPi . sense object using PIR sensor and update timing in SQLite db also publish msg “PIR object detected” on MQTT topic name :: “PIR_Sensor” and display time on screen

| PIR |  | RPi |  |
| --- | --- | --- | --- |
| GND |  | 6 |  |
| VCC |  | 2 |  |
| OUT(DATA) |  | 12 |  |
| If using LED |  |  |  |
| LED |  |  | RPi |
| - |  |  | Ground |
| + |  |  | Output-18 |
| PIRpy : |  |  |  |
| import RPi.GPIO as GPIO |  |  |  |
| import time |  |  |  |
| import sqlite3 |  |  |  |
| import paho.mqtt.client as mqtt |  |  |  |
| GPIO.setmode(GPIO.BCM) |  |  |  |
| GPIO.setup(24,GPIO.OUT) |  |  |  |
| GPIO.setup(18,GPIO.IN) |  |  |  |
| connection = sqlite3.connect('test.db') |  |  |  |
| cursor = connection.cursor() |  |  |  |
| cursor.execute('''CREATE TABLE pirTB ( |  |  |  |
| sensorData VARCHAR(100), |  |  |  |
| currentTime TIMESTAMP);''') |  |  |  |
| insertQuery = """INSERT INTO pirTB |  |  |  |
| VALUES (?, ?, ?);""" |  |  |  |

while(True):

myin=GPIO.input(18)

if myin==True:

print("Motion Detected")

cursor.execute(insertQuery, (“Motion Detected”, currentDateTime))

connection.commit()

broker_url = "broker.emqx.io"

broker_port = 1883

client = mqtt.Client()

client.connect(broker_url,broker_port)

client.publish(topic="PIR_Sensor", payload=”motion detected”, qos=0, retain= False)

time.sleep(1)

GPIO.output(24,True)

time.sleep(0.5)

GPIO.output(24,False)

time.sleep(0.5)

cur.execute("select * from pirTB ");

ans=cur.fetchall();

for row in ans:

print(ans);

connection.close()

-----------------------------------------------------------------------------------------------------------------

ADD Data in CSV File

import csv

data = [

['Albania',545822,'AL','ALB'],

['American Samoa',56335,'AS','ASM'],

['India',3524,'IN','IND']

]

header=['name','area','countrycode2','countrycode3']

with open('countries.csv','a',encoding='UTF8') as f:

writer=csv.writer(f)

#write the header

writer.writerow(header)

#write data

#writer.writerow(data)

for i in range(0, len(data)) :

writer.writerow(data[i])

f.close()

rows=[]

with open('countries.csv','r') as file:

csvreader=csv.reader(file)

header1=next(csvreader)

for row in csvreader:

rows.append(row)

print(header1)

print(rows)

file.close()

sqlLITE commands

---------------------------------------------------

sqlite 3 install and use

---------------------------------------------------

install: sudo apt-get install sqlite3

creating new database or entering into existing database: sqlite3 newDB.db

creating table: create table tableName (id integer, name text);

show table: .table

insert: insert into demo (demoID, demoName) values(1, "a1");

show: select * from demo;

.exit or .quit to exit

Node js with mqtt

/*install following command*/

npm init -y

npm install mqtt mosca

/*Replace the following condition in node_modules/jsonschema/lib/validator.js  at line no 109*/

if((typeof schema == 'boolean' && typeof schema == 'object') || schema === null){

throw new SchemaError('Expected `schema` to be an object or boolean');

}

/*Create broker.js*/

//============================broker.js==================================

//MQTT Broker

//Help us to create mqtt

var mosca = require("mosca");

var settings = {port:1234};

var broker = new mosca.Server(settings);

broker.on("ready",()=>{

console.log("Broker is ready....");

});

==========================================================================

//============================publisher.js=================================

var mqtt = require("mqtt");

var client = mqtt.connect("mqtt://localhost:1234");

const TOPIC = "MY_TOPIC"

const message = "Hello there I am sending the message";

client.on("connect",()=>{

setInterval(()=>{

client.publish(TOPIC,message);

console.log(`Message Received ${message}`)

},3000);

});

==========================================================================

//============================subscriber.js================================

var mqtt = require("mqtt");

var client = mqtt.connect("mqtt://localhost:1234");

var TOPIC = "MY_TOPIC"

//client connect with broker then it will subscribe

client.on("connect",()=>{

client.subscribe(TOPIC);

});

//Sending the message

client.on("message",(topic,message)=>{

message = message.toString();

console.log(`Message sent : ${message}`);

});

==========================================================================

mqtt settings:

mqtt client name: client (any)

protocol: mqtt / tcp

host: localhost:1234  (the one that is in the code)

normally host is: broker.emqx.io

when node broker.js will run, then only the connetion will eshtablish and it will show connected in mqtt box or else it will show connection error

//Run the files in different terminal

node broker.js

node publisher.js

node subscriber.js